Jump Start System

- Turn off ignition and all accessories, put vehicle in Park, set emergency brake

- Make sure Jump Start System is OFF.

- Remove booster clamps from protective holsters, squeeze and pull down.

- Connect positive (+) red clamp to battery’s positive terminal.

- Connect negative (-) black clamp to chassis or a solid non-moving metal vehicle component.  If audible alarm sounds, polarity is not correct, reverse the connections.

# DO NOT TURN ON SAFETY SWITCH

## WITH ALARM SOUNDING

- Turn Jump Start System’s power ON.

- Start vehicle (do not crank for more than 5-6 seconds).

- After vehicle starts, turn jump start system OFF, disconnect negative (-) black cable followed by the positive (+) red cable, and return to holsters.

- Return the jump start system to the library.

Reviewed by the Board of Trustees

April 18, 2006